Notes on recent changes in SFIT2, versions 3.5 - 3.7

Effective Apodisation Function (from v3.5 release notes Jan 97)

The new empirical aopdization parameters required by the Binput file

are identified in the Detail file. They are:

ieap: the empirical apodization code

ieap=0 : none

ieap=1 : a tabular function is read in from unit 19, eapfil

ieap=2 : a polynomial of order neap; coefficients read from unit 19

ieap=3 : a Fourier series of order neap; coefficients from unit 19

neap: see above; only used if ieap=2 or 3

irteap: empirical apodization retrieval flag;

if 1, empirical  apodization parameters are retrieved;

if 0, they are not.

Only used if ieap=2 (polynomial).

If irteap=1, the coefficients in  unit 19 are used as the a priori values for the retrieval.

eappar: if irteap=1, this is a scale factor which will be applied to the a

priori coefficients from unit 19.

seappar: the uncertainty in eappar (only used if irteap=1).

Effective Apodisation File (Eapfil)

This file contains parameters to define the specific apodization

function as specified by ieap. Its name is read from the fifth line

of Binput. If the empirical apodization parameters are being retrieved

(irteap=1), the information in this file is the a priori. Otherwise

the information is used only by the forward model in imposing the

specified apodization.

The structure of the file is:

jeap

eapf(1) eapf(2) ... eapf(jeap)

[eapx(1) eapx(2) ... eapx(jeap)]

jeap simply specifies the length of the following vectors. eapf is the

vector of apodization parameters, whose meaning depends on ieap:

ieap=1 : eapf contains the values of the apodization function eapdz;

in this case eapx contains the values of path difference at

which the function is specified.

ieap=2 : eapf contains the coefficients of a polynomial, defined

by

eapdz = 1 + (eapf(1)-1)*(x/xmax) + (eapf(2)-1)*(x/xmax)^2 + ...

where x is the path difference

ieap=3 : eapf contains the coefficients of a Fourier series

eapdz = 1 + (1-eapf(2))*sin(2*pi*eapf(1)*x/xmax) +

(1-eapf(3))*cos(2*pi*eapf(1)*x/xmax) +

(1-eapf(4))*sin(4*pi*eapf(1)*x/xmax) +

(1-eapf(5))*cos(4*pi*eapf(1)*x/xmax) + ...

eapx is read only when ieap=1.

Features added in SFIT2 v3.6

The rdrv.ctl file now contains a activation flag and filename for the K file.  Both are at the end of the rdrv file.  If the flag is set true, then the filename must be specified.  Since there  are now values after the output spectra name, the output spectra name should only appear if the retrieval flag is set to false.  Before the output spectra name could be left on the end of the file with no problem.

New lines in binput have been added to specify the binary files from which the ascii spectral file will be derived.  On the same line as the spectrum code (SZA*100) there is now a parameter that gives the number of bnr files for that spectrum code, which can be from 1 to 3.  Following that the names of the bnr files are given, one per line. NOTE: the RDRV program does not actually use this information, it is up to the user to arrange to have the ascii spectral file available with the name as specified in rdrv.ctl.  This feature was added to allow use where a separate program to perform the bnr to ascii conversion can get all the information it needs from one file, the binput file.  In this mode of operation, the same ascii file name is often used for many retrievals, each overwriting the previous one.  If you can arrange the correct ascii spectral file to be available without using this new information in binput, then so be it.

SFIT2 v3.74 (January 2000)

This adds several new capabilities to those available in v3.6.

1. allows input of an arbitrary, tabular phase function analogous to the tabular apodization function which was introduced in v3.5. Using these 2 functions together, it is now possible to use a measuredILS function in the forward model.

2. allows input and/or retrieval of the parameters of a polynomial phase function. This capability is handled analogously to the polynomial apodization function already available (also introduced in v3.5).

3. allows specification of a 'zero level' for the calculated spectrum.  Previously, the zero level could be retrieved but not fixed at a retrieved value in a subsequent run.

SFIT2 v3.74 was developed from v3.6 so that input files for v3.6 can be used as a starting point to build v3.74 inputs. The only changed input file is the binput. In addition, there is a new file for the empirical phase function.

Binput changes:

The sixth line of the binput file is new. It names the file containing the empirical phase function data. This is the  variable 'ephsfil' in the Fortran code.

In about the middle of the file is the line of forward model parameters:

1.00000    257.143    3.863636   0     1     0    2    2   2   2

The last two parameters are new. They are called 'iephs' and 'nephs', and serve the same role in the empirical phase function as the two parameters immediately preceding them (ieap and neap) do in the empirical apodization function. Namely,

iephs - empirical phase error code

iephs=0 no empirical phase function

iephs=1 a tabular function is read in

iephs=2 a polynomial of order nephs

nephs - see iephs

Next there is one new parameter in the line of retrieval switches:

1 2 0 0 0 1 1

This last is 'irtephs', and is analogous to the parameter preceding it, irteap.

irtephs - 0=use empirical phase in FM, do not retr. params.

1=retrieve empirical phase parameters

(only available if iephs=2)

Finally, in the line containing the parameters for a given microwindow,

1167.5300 1167.6500    4    0.  0

there is one new value. It is 'zshift' and is the offset to be applied to the spectrum in that window

(the 'zero level'). Note that this value is always used, whether you are fitting the zero level or not.

The last parameter in that line is as before. If it = 0, the zero level in that microwindow is not fit. If

it =1, then it is fit.

Empirical phase file

The format of empirical phase files is the same as those specified for empirical apodisation files, except the option of Fourier coefficients is not implemented)

Here are 2 examples of empirical phase files:

Sample file starts and ends at dashed lines:

--------------------------------------

2

0.90000      0.90000

--------------------------------------

This first file contains 'jephs', the number of parameters to be read in, followed by the vector

'ephsf(jephs)'. In this case a quadratic phase function is specified with both the linear and quadratic

coefficients = 0.9.

Sample file starts and ends at dashed lines:

-----------------------------------------------------------------------------

11

0.  .01  .02  .03  .04    .05  .06    .07  .08   .09   .10

0.  25.  50.  75.  100.  125. 150. 175. 200. 225. 250.

-----------------------------------------------------------------------------

This file defines a function with 11 tabular points. The values of the function are 0., .01, etc. The  third line specifies the OPD values at which the function is specified. So for example, at 150. cm OPD, the phase is 0.06.